Norwegian Kickboxing Federation

Approval criteria for protection equipment in competitions

In General for all equipment

1. Protective materials in the equipment shall be of one of or a combination of the following shock-absorbing materials:

a. Polyurethane open cell foam.

b. EVA Ethylene Vinyl Acetat.

c. Latex foam, natural or synthetic.

d. If other materials of adequate quality are used (other than those listed above), they will be assessed individually and case by case.

2. Protective materials of all equipment shall be in one piece (can be of several different materials) and of a type / construction that can not be displaced.

3. All equipment should be CE approved and marked. If no CE approval the equipment can still be approved by the Committee if it otherwise meets all requirements, but should be assessed more stringent.

4. Manufacturers, importers and suppliers, kindly send the relevant documentation (Including product name, type, variant / versions, CE approval and / or other relevant documentation).

5. Lack of relevant documentation may cause the equipment not being approved or rejected.

Gloves full and light contact:

1. Weight should be 10 oz (285g), with no greater deviation than + / - 5%.

2. Protective materials should weigh at least half of the total weight of the gloves.

3. Protective materials to be in one piece (can be of several different materials) and of a type/construction that can not be displaced.

4. Protective materials shall be at least 40 mm thick in contact areas.

5. Area of contact shall be marked with white and be of a quality that does not peel off or is harmful to the eyes.

6. The thumb to be sewn permanent and fasten. Furthermore, it shall be cast / designed to be rounded and / or located under the glove, not so that it sticks out or can easily cause cuts or eye damage.

7. The glove shall have a closing device without sharp / uneven surface which can cause injuries. This also applies to the glove as a whole.

Gloves semi contact:

1. Weight shall be at least 6 oz (171g), with no greater deviation than + / - 5%.

2. Protective materials should weigh at least half of the total weight of the gloves.

3. Protective materials to be in a piece (may be of several different materials) and of a type/construction that can not move around.

4. Protective materials shall be at least 30 mm thick in the contact area.

5. The thumb should be made permanent and fasten. Furthermore, it shall be cast / designed to be rounded and / or located under the glove, not so that it sticks out or can easily cause cuts or eye damage.

6. The glove shall have a closing device without have sharp / uneven surfaces which can cause injuries. This also applies to the glove as a whole.

Shin protection:

1. Protective materials to be in one piece (can be of several different materials) and of a type/construction that can not be displaced.

2. Protective materials shall be at least 25 mm thick in the Primary contact area.

3. The shin protection should have reinforcement over the lower leg bone and should cover the largest possible part of the lower leg bone (above the foot protector) and well around the leg.

4. The shin protector should have an attachment device which keeps well fasten to the leg and without sharp / uneven surfaces which can cause injuries. This also applies to the protector as a whole.

Shin/Instep protector:

1. Protective materials in both the leg and instep portion of the protector to be in one piece

(may be of several different materials) and of a type / construction that can not be displaced.

2. Protective materials of the shin should be at least 25 mm thick in the primary contact area, while the instep section should be at least 20 mm thick in the primary contact area.

3. Shin/Instep protector to be in one piece and have good cushioning in the transition area between the leg and wrist, but still be movable / flexible in relation to the exercise of various kickboxing leg techniques.

4. Shin part of the protector should have reinforcement over the lower leg bone and should cover the largest possible part of the lower leg bone (above the instep and foot section) and well around the leg.

5. Instep part of the protector should cover at least 70% of the instep. This includes the ankle, instep and wide, and well down on the floor.

6. Shin/Instep protector should have an attachment device which keeps it firmly to the leg / ankle and without sharp / uneven surfaces which can cause injuries. This also applies to the protector as a whole.

Foot Protectors:

1. Weight shall be at least 6 oz (171g).

2. Protective materials should weigh at least half of the protection's overall weight.

3. Protective materials to be in one piece (can be of several different materials) and of a type/construction that can not be displaced.

4. Protective materials shall be at least 25 mm thick in the Primary contact area.

5. Foot protector to cover the entire foot. This includes the ankle, wide and toes, as well as down to the floor around the entire foot.

6. The foot protector should have an attachment device which keeps it firmly to the foot and without sharp / uneven surfaces which can cause injuries. This also applies to the protector as a whole.

Head Protectors:

1. Weight shall not exceed 450g.

2. Protective materials should weigh at least half of the protection's overall weight.

3. Protective materials to be in one piece (can be of several different materials) and of a type/construction that can not be displaced.

4. Protective materials should be minimum 25 mm thick at the contact area.

5. The head protector should have padding / protection of the skull roof.

6. It also should have an attachment device which keeps it firmly to the head, and without sharp / uneven surfaces which can cause injuries, both on the outside and inside.